### Predictive Maintenance of Metro Train Compressors Using Multivariate Sensor Data

Use Metro_Train_Maintenance dataset from classification.db for this problem.

### Background

Metro trains rely on Air Production Units (APUs) to maintain pneumatic pressure for braking, suspension, and other subsystems. Failures in this unit can result in major operational disruptions, increased maintenance costs, and passenger safety risks. Anticipating such failures in advance enables condition-based maintenance, reducing unplanned downtime.

The MetroPT-3 dataset captures over 1.5 million time-stamped sensor readings from compressor systems onboard operational metro trains. It includes both analog (e.g., pressure, temperature, motor current) and digital (e.g., valve states, switches) signals collected at a 1-second frequency over several months.

However, this dataset does not include a ready-made target label for failure prediction. Learners are expected to engineer this label from known failure events and use it to build a robust classification model.

### Objective

Your task is to build a binary classification model that predicts whether a compressor failure will occur within the next 60 seconds, based on real-time sensor readings. This simulates a real-world predictive maintenance system that alerts operators before a failure occurs.

### Dataset Overview

- Rows: 1.5 million+

- Features: 15 sensor signals (pressures, temperatures, valve states)

- Timestamp column: timestamp

- Target column: failure_in_60sec (you will create this)

- Label distribution: Highly imbalanced (failures are rare)

- Unnamed: 0
 Row index representing time steps (recorded every 10 seconds)

- timestamp
 Observation timestamp in YYYY‑MM‑DD HH:MM:SS format

- TP2 (bar)
 Pressure measured at the compressor

- TP3 (bar)
 Pressure at the pneumatic panel

- H1 (bar)
 Pressure generated when the cyclonic separator filter discharges

- DV_pressure (bar)
 Pressure drop when the dryer discharge valve operates (zero indicates compressor under load)

- Reservoirs (bar)
 Downstream air reservoir pressure (should align with TP3)

- Oil_temperature (°C)
 Temperature of the compressor oil

- Motor_current (A)
 Current consumed by the compressor motor (varies from 0 A to ~9 A depending on load state)

- COMP
 Compressor intake valve electrical signal (binary: 1 = offloaded or off; 0 = under load)

- DV_eletric
 Discharge valve control signal (binary: 1 = under load; 0 = off or offloaded)

- Towers
 Tower dryer operation indicator (binary: 0 = tower 1 active; 1 = tower 2 active)

- MPG
 Main pressure gauge signal (binary: triggers when pressure drops below ~8.2 bar)

- LPS
 Low pressure sensor signal (binary: activates when pressure drops below ~7 bar)

- Pressure_switch
 Signal indicating the dryer tower discharge activation (binary)

- Oil_level
 Oil level sensor signal (binary: 1 indicates low oil level)

- Caudal_impulses
 Pulse count from the airflow sensor measuring air flow from APU to reservoirs

### Task Instructions

#### 1. Create the Target Column: failure_in_60sec

You are given a set of known failure periods from the dataset documentation:

| Failure | Start Time | End Time |
| --- | --- | --- |
| #1 | 2020-04-18 00:00 | 2020-04-18 23:59 |
| #2 | 2020-05-29 23:30 | 2020-05-30 06:00 |
| #3 | 2020-06-05 10:00 | 2020-06-07 14:30 |
| #4 | 2020-07-15 14:30 | 2020-07-15 19:00 |

### Project Steps to Create a Target Feature from the MetroPT3 Dataset

- Load the Dataset:

  - Import the necessary libraries: pandas and numpy.

  - Load the dataset using pd.read_csv(), ensuring the timestamp column is parsed as dates.

- Define Known Failure Windows:

  - Create a list of tuples representing the start and end times of known failure events.

  - Convert these string tuples to pandas datetime objects for easier manipulation.

- Extract Context Around Failures:

  - For each defined failure window, extract data from one hour before to ten minutes after the failure event.

  - Create a new DataFrame that combines these extracted data chunks while removing duplicates.

- Create a Failure Label Column:

  - Add a new column called failure to the DataFrame, initializing with 0 (indicating no failure).

  - Update this column to 1 for timestamps that fall within the defined failure windows.

- Label Future Failures:

  - Create another column called failure_in_60sec that indicates whether a failure occurs within the next 60 seconds.

  - Use a rolling window of size 60 to check for any future failures and update this column accordingly.

- Remove Intermediate Failure Labels (Optional):

  - Drop the failure column if you only want to keep the failure_in_60sec label for your predictions.

- Sample Non-Failure Data:

  - Create a mask to filter out timestamps that are within one hour before and after any failure window.

  - Sample 170,000 rows from the non-failure regions to create a balanced dataset, ensuring these are labeled with failure_in_60sec set to 0.

- Combine and Shuffle the Datasets:

  - Concatenate the failure context data and the sampled normal operating data into a single DataFrame.

  - Shuffle the combined DataFrame to ensure randomness in the data for training purposes.

#### 3. Modeling

- Split into train-test sets (e.g., 80-20).

- Choose the best model for training by trying out with different models and scale data if required.

- Evaluate using:

  - Accuracy

  - Precision, Recall, F1-score

  - Confusion Matrix

  - ROC AUC Score

- Analyze feature importance to interpret which sensors contribute most to failure prediction.

### Deliverables

Your final notebook should contain:

- Data preparation and target labeling logic

- Class distribution analysis after sampling

- Model training and evaluation

- Feature importance plots

- Business interpretation of model performance

### Business Impact

An accurate early-warning model allows metro operators to:

- Schedule targeted inspections or repairs before failures occur

- Prevent operational delays or safety incidents

- Transition from reactive to condition-based maintenance

This project simulates a real industrial ML deployment workflow for predictive maintenance.